Digital Projects, LP3 2001

Department of Information Technology,

Lund Institute of Technology

Supervisor: Bertil Lindvall

2001-03-07

#### TETRIS

Technical report

Calle Grunditz and

Anna Lindholm

Group 24

Abstract

This report describes the process of constructing a Tetris-game - from designing the hardware to beta-testing the final version of the software. The main part of the hardware is the processor, in this case a MC68008 with 48 pins, which communicates with the peripheral components. Programmable logic, more precisely PALCE22V10-components were used to make the construction more effective. The software was written in C/Assembler with a development system for MC68008 called “it68”, which consists of a hardware emulator and a software making it possible to communicate with the hardware using a computer.

Contents

Introduction	4

Hardware construction	5

Components	5

Processor – MC68008	5

PAL – PALCE22V10	5

Program memory – EPROM 27C128	5

Memory – SRAM 6264	5

Writeable memory – EEPROM X2816C	5

Display – Batron 128064B	5

Buttons	6

Oscillator – 8 MHz	6

Latch - 74HC73	6

Connection of the components	6

Address map	6

Building and testing the hardware construction	6

Software	8

tetris.cmd	8

exp2.68k	8

avben.68k	8

pmain.68k	8

tetris.c	8

game.c	8

display.c	9

Problems	10

Conclusions	11

References	12

Appendix A – scheme over the hardware construction	13

Appendix B – the PAL programs	14

Adress.pld	14

IO.pld	14

Avbrott.pld	15

Appendix C – the c and assembly programs	16

Tetris.c	16

Display.c	17

Game.c	18

Avben.68k	30

exp2.68k	31

Pmain.68k	31

Appendix D – how to play the game	33

# Introduction

The project was an assignment in the course ”Digital projects”. The purpose of the course is, freely translated from the homepage of the course[1], ”to illustrate industrial development. The project aims towards being a prototype for further development with necessary documentation. The main part of the course consists of constructing, building and testing the construction.”

The goal of the project was to construct a Tetris-game using a M68008 processor. The specifications were as follows:

- The graphic will be presented on a LCD display and a user will be able to control the game with a control unit.

- There will be seven different sorts of blocks in the program and they will look like this:

- When a new block is requested, it will be randomly chosen.

- The player controls the blocks with buttons. It will be possible to rotate the blocks and move them to the right, left and down. There will also be buttons that start the game and reset it.

- When a block is moved to the side it will stay there and not move around to the other side.

- When a line is full it is removed and the lines above are moved down, with a score bonus of 10 points per row. When a block reaches the bottom the player will get 1 point. For every 200th point the speed will increase until a maximum level is reached. The scores will be visible on the display.

- The game should be 8 chars wide and 16 chars high.

- If there is time the game will include a high score-list, which saves the ten best results and the initials of the players.

In the first chapter the hardware is described, in the second chapter we describe the software, after that the problems and conclusions are discussed. At last there are some appendixes with the scheme over the hardware construction, the program code and how to play the game.

# Hardware construction

## Components

Data sheets to all components that are possible to use in this course can be found in the information bank on the homepage of the course[2].

### Processor – MC68008

The available processors to choose from were the MC68008 and MC68HC11, of which the MC68008 seemed more interesting and allowed for more freedom and actual constructing. The MC68008 works in an asynchronous mode as default but can be triggered to work with synchronous components as well. It has 20 address bits and 8 data bits.

### PAL – PALCE22V10

Programmable logic is used extensively because of its advantages in making the constructing easier, and more precisely was the PALCE22V10 selected for this task. This logic unit will be programmed in pldasm, which is described on course’s homepage[3]. PALCE22V10 has 22 pins of which all can be used as inputs and 10 as outputs.

During the design stage, three PAL units seemed to be necessary. The first one decodes the addresses and sets the control signals to the peripheral units (i.e. display, EPROM, EEPROM and SRAM) and also sends the DTACK signal to the processor. The second PAL takes care of the interrupts. Later in the project a change was made which caused interrupts to no longer be needed and thus this PAL unit is not used, but the unit still exists in the construction. The third PAL handles the I/O units, i.e. the buttons. The PAL programs can be seen in Appendix B.

### Program memory – EPROM 27C128

The program code for the processor has to be saved in a memory. 27C128 is a 16 k * 8 bit parallel EPROM and 16 kbytes seemed enough for this particular construction. 27C128 is controlled by three signals /G and /E which can be compared to the more common signal names /OE and /CE which mean output enable and chip enable.

### Memory – SRAM 6264

During the execution, the program uses memory temporarily. 6264 has 8 k * 8 bit parallel CMOS SRAM, which is large enough for the program. 6264 is controlled by /CS1, CS2, /OE and /WE which mean chip select, output enable and write enable.

### Writeable memory – EEPROM X2816C

Since the program has a high score-list it needs a memory that can be changed by the program without loosing its information when the power is turned off. The high score-list only needs a few hundred bytes, and the smallest available EEPROM was the X2816C which has 2 k * 8 bit parallel memory. X2816C is controlled by /CE, /OE and /WE which mean chip enable, output enable and write enable.

### Display – Batron 128064B

For the visual output a display is needed. The display recommended for this purpose was a Batron 128064B. The display was retrieved from the department of Information Technology with some additional components connected to it to protect from disturbance. The display has a window of 128 * 64 pixels, with two CS signals controlling a grid of 64 * 64 pixels each. It is possible to both read from and write to the display, through the setting of a R/W signal. E is an enable signal controlled by the processors own E signal. The D/I-input controls if there is data or if there is an instruction coming on the data inputs. The display has 8 data inputs, which implies that you need to write 8 pixels at one time.

### Buttons

The blocks in the game are to be moved when the player presses a button. Regular on/off buttons are used for this purpose.

### Oscillator – 8 MHz

The processor needs an external clock. An oscillator, described on the course homepage[4], was constructed using a crystal with the frequency 8 MHz.

### Latch - 74HC73

Two JK-latches synchronizes the processor with the peripheral and indicates that there is a valid address on the address bus.

## Connection of the components

The whole hardware construction can be seen in Appendix A.

Our hardware construction uses the address bus to determine which component that will be used at the moment. The different components use different parts of the address area. The address bits A13 to A16 control which unit that is in use at the moment, the lower address bits (A0 to A12) control which part of the selected unit that will be used.

When e.g. the EEPROM is accessed, the address bus set the address to this unit, then the address decode PAL decodes that address and activates the control signals to the EEPROM and at the same time deactivates the control signals to all the other peripheral units. When a peripheral unit is deactivated it must set the data bus signals to the state ”Z”. If it does not, there will be a collision on the data bus and it won’t work as intended.

## Address map

The following figure shows the memory-map.

Address:

0	4000	8000	c000	10000	14000	18000	1c000

E

P

R

O

M

E

E

P

R

O

M

S

R

A

M

D

I

S

P

L

A

Y

1

D

I

S

P

L

A

Y

2

B

U

T

T

O

N

S

## Building and testing the hardware construction

The hardware has been built on a card specially made for this course. The components are placed in sockets and have mainly been connected to each other by a winding technique. A small cable is fastened on the pins of the socket with a tool winding the cable around the pin. Some parts, mainly the ground and the power, had to be solded to the construction.

Once the hardware was built it could be tested with a development system called it68. On the course homepage there is a description of this development system in Swedish[5]. This system consists of a hardware emulator that can communicate with a PC. The program in the PC can set what addresses will address the emulators internal memory and what addresses that reference the hardware construction. Thus it is possible to test each part of the hardware at a time. When the software is written it is possible to load this into the emulator and use all other addresses on the hardware construction to test the program.

# Software

The software is written in C, all the program lists can be seen in Appendix C. We have split the program into some parts that are described below.

## tetris.cmd

In this file some important information for the compiler is set.

## exp2.68k

‘exp2.68k’ is written in assembly code and controls the interrupts at level 2.

## avben.68k

‘avben.68k’ is an assembly program that allows interrupts at level 4 and higher. We don’t allow interrupt 2 since we generate, but don’t use it. You can read more in ‘tetris.c’.

## pmain.68k

This is the main program, which initiates the stack pointer, the global data pointer and the interrupt vector. It also resets all registers and makes a subroutine-call to _main.

## tetris.c

‘tetris.c’ is the main program file and does also handle the I/O-functions (the buttons). In the beginning it was thought to handle the interrupts as well, but since interrupts aren’t used, these parts of the program are never referenced. An interrupt is generated if a button is pressed down, but in practice this means very many interrupt calls every time a button is pressed and i.e. the blocks were moved several steps at a time. Now the buttons are checked on demand by the program when nothing else is done. This works without any problems - the brick moves and rotates in the way the player want and the program won’t slow down when a button is pressed. The time between checking buttons can easily be changed.

## game.c

‘game.c’ holds the code that handles the main functions concerning the actual game. A four-dimensioned matrix represents the seven blocks and their rotations. The first place tells what block it is, the second one the rotation and the two last positions hold the relative x and y position, giving the vector the total size 7 * 4 * 4 * 2. To make the program easier, all blocks have four rotations even if they are not all visible for the player.

A two-dimensioned vector with size 9 * 17 represents the game board with its 8 * 16 squares; we don’t use the zero position. Every element in this vector is either full or empty. It’s full if there is a block in the square and empty if not. The block being moved by the player at the moment isn’t represented in this vector, it only contains the blocks that are placed earlier on. The procedure for moving a block follows: The block's x- and/or y-coordinates are changed, and the program checks if it is possible to have the blocks at that spot or if there is a collision. If the move is valid the block is moved on the display otherwise the block is moved back to its last place. If the block lands on top of another block, it stays there and can’t be moved anymore. The game-board is searched for full rows and if any are found, the lines are removed and everything above is moved down one row.

## display.c

The display is made of two parts, each 64 * 64 pixels or 8 * 8 characters big, where each character is 8 * 8 pixels. In the functions that write chars, x is positive to the right and y positive down. The upper left corner thus has the coordinates (1, 1) and the lower right corner has the coordinates (8, 16).

# Problems

During the course of the construction, several problems arose. The most trouble and confusion were caused by interrupts. It wasn’t too easy to understand in what way they worked. After a long time the decision was made to use interrupts whenever a button was pressed. When the software was written we found that the interrupts didn’t work properly as intended for our construction so we changed it and in the final program the buttons are checked on a regular basis. The hardware is constructed to be able to handle interrupts and can easily be implemented in the software. There is an advantage to not removing it; if someone would like to write another kind of game for the hardware, they have the possibility to use interrupt calls. One can have several EPROMs with different games.

Another confusing problem was with the PAL code for interrupts - one signal was inverted, causing the processor to work synchronously all the time except for when using the display. That didn’t work at all since the display needs to work in synchronous mode. After a while the mistake was realized and the PAL program was corrected.

The high-score list has a weird bug in it, causing one of the initials on the first row to disappear, but since the specifications included a high score-list if time allowed it, and since it works almost correctly, not much effort has been made to remove the bug.

# Conclusions

Naturally, there are possible improvements to the c code, e.g. we could have made a function with a string as input parameter, which writes the string to the display. If a larger display had been available we could have made the game according to the standard version with a size of 10*20 squares, and showing the next block. The user interface could be better and the letters could look nicer, but we aren’t designers and didn’t have the time to do it better than it is now. The memory usage could have been decreased if all graphic data had been moved to the EEPROM memory area. Since we didn’t use interrupts, two PAL units would probably have been enough. The scores decided through the specifications turned out to be not so good and are changed in the implementation, yet some changes could be made to make the game more enjoyable.

Digital Projects is a very educational and fun course where you are given a chance to apply your theoretical knowledge at the same time as you construct something useful. We have learned a lot about the process of hardware construction. Our final product works as we want to and we are satisfied with it. We think that the goal of the course is achieved.

# References

- The homepage of the course: http://www.it.lth.se/it/courses/Digp/index.html

- Data sheets in the information bank: http://www.it.lth.se/it/courses/Digp/datablad/datablad_index.html

- Oscillator information: http://www.it.lth.se/it/courses/Digp/PDF_files/oscillators.pdf

- Pldasm: http://www.it.lth.se/digproj/PDF_files/pldasm.pdf

- It68 utvecklingssystem för MC68008 (in Swedish only): http://www.it.lth.se/digproj/PDF_files/it68.pdf

# Appendix A – scheme over the hardware construction

# Appendix B – the PAL programs

### Adress.pld

' Adressavkodning, (PALCE22V10)

' Input:  1-13

' Output: 14-22

device 22V10

A13     1

A14     2

A15     3

A16     4

RW      5	' aktivt låg

DS      6	' aktivt låg

AS      7	' aktivt låg

VMA	8	' aktivt låg

DTACKIO	9	' data-acknowledge från I/O-enheten

CSEEPROM        14 ' aktivt låg

CSEPROM         15 ' aktivt låg

CS1SRAM         16 ' aktivt låg

OEMEM           17 ' aktivt låg

WEMEM           18 ' aktivt låg

CS1DISP         19 '

CS2DISP         20 '

RWDISP          21 '

DTACK           22 '

VPADECODE	23 ' VPA-decode is an active high indicating that /AS has been asserted and

' the address bus is addressing an M6800 peripheral.

' .. alltså: en form av Chip Select för displayen.

start

CSEPROM  /=        /A14 * /A15 * /A16 * /AS; 		' 0 - 3FFF

CS1SRAM  /=  A13 *  A14 * /A15 * /A16 * /AS; 		' 6000 - 7FFF

CSEEPROM /=  A13 * /A14 *  A15 * /A16 * /AS; 		' A000 - BFFF

CS1DISP  /= /A13 * /A14 * /A15 *  A16 * /AS * /VMA; 	' 10000 - 11FFF

CS2DISP  /=  A13 * /A14 * /A15 *  A16 * /AS * /VMA; 	' 12000 - 13FFF

OEMEM /= /DS * RW;

WEMEM /= /DS * /RW;

DTACK /= /CSEPROM + /CS1SRAM + /CSEEPROM + DTACKIO;

'RWDISP = RW * /CS1DISP + RW * /CS2DISP; ' ska faktiskt alltid vara låg.

VPADECODE = /A14 * /A15 *  A16 * /AS; ' ChipSelect för displayen

End

### IO.pld

' IO, (PALCE22V10)

' Input:  1-13

' Output: 14-22

device 22V10

BUTTON1 1

BUTTON2 2

BUTTON3 3

BUTTON4 4

BUTTON5 5

BUTTON6 6

A13     7

A14     8

A15     9

A16     10

D00     14	' status för button 0 skickas ut på databussen D00

D01     15

D02     16

D03     17

D04     18

D05     19

INTREQ  20	' begär avbrott 2

DTACKIO	21	' data skickas på databussen

'PAL:en gör två saker:

'1. Om en knapp trycks ner aktiveras INTREQ-signalen in till avbrottshanteraren.

'2. Då rätt address kommer på addressbussen skickas information om vilka knappar

'som tryckts ned till databussen.

'Addressen för knapparna är 16000-17fff

start

DTACKIO = A13 * A14 * /A15 * A16; ' knapparnas address finns på addressbussen

' om rätt address kommit på addressbussen är det fritt fram att skicka ut

' knapparnas status på databussen

D00 = BUTTON1 * DTACKIO;

D01 = BUTTON2 * DTACKIO;

D02 = BUTTON3 * DTACKIO;

D03 = BUTTON4 * DTACKIO;

D04 = BUTTON5 * DTACKIO;

D05 = BUTTON6 * DTACKIO;

D00.ENA = DTACKIO; ' three-state, Z om fel address

D01.ENA = DTACKIO;

D02.ENA = DTACKIO;

D03.ENA = DTACKIO;

D04.ENA = DTACKIO;

D05.ENA = DTACKIO;

' skicka begäran om avbrott ifall en knapp tryckts ner

INTREQ = BUTTON1 + BUTTON2 + BUTTON3 + BUTTON4 + BUTTON5 + BUTTON6;

End

### Avbrott.pld

' Avbrottshantering, (PALCE22V10)

' Input:  1-13

' Output: 14-22

device 22V10

FC2     1

FC1     2

FC0     3

VPAIN   4 ' VPA från synkkretsen

INTREQ  5 ' avbrott begärt av styrenheten?

AS      6 ' aktiv låg

IPL1    14 ' aktiv låg

IPL20   15 ' aktiv låg

VPA     16 ' aktiv låg

start

' Tabell över IPL0/1/2 finns på sid 107 (IT-68)

IPL1 /= INTREQ; 'genererar avbrott 2 om INTREQ hög

IPL20 = /INTREQ + INTREQ; 'ska alltid vara hög

' Då processorn har uppfattat IPL0/1/2-signalen blir FC0/1/2 samt AS aktiva,

' och då sätts VPA låg.

'VPA sätts låg om VPAIN aktiv eller en IACK-signal från processorn (s.107)

VPA /= /VPAIN + /AS * FC0 * FC1 * FC2;

end

# Appendix C – the c and assembly programs

### tetris.c

#option separate led_reg   /* Ett satt att definiera adresser */

#option separate button_reg

#option separate display1_instr

#option separate display1_data

#option separate display2_instr

#option separate display2_data

#define TRUE 1

#define FALSE 0

short int led_reg, button_reg;         /* I tetris.cmd far led_reg sitt varde */

short int display1_instr, display1_data;

short int display2_instr, display2_data;

unsigned char *disp_mem;

unsigned char *eeprom;

int control_unit[6]; /* 6 knappar */

void addToControlBuffer(int num);

void check_buttons();

void resetControls(void);

int isButtonPushed(int num);

#include "display.c"

#include "game.c"

main()

{

disp_mem = (unsigned char *) 0x6000;

eeprom = (unsigned char *) 0xa000;

_avben();                 /* tillat avbrott */

display_on();

game_start();

}

exp2() {

/* avbrott 2 */

return(0);

}

void check_buttons()

{

/* lägg till i teckenbuffern om knapp tryckts ner */

int i = button_reg;

if ((i & 1) == 1)  /* knapp 1? */

addToControlBuffer(1);

if ((i & 2) == 2)  /* knapp 2? */

addToControlBuffer(2);

if ((i & 4) == 4)  /* knapp 3? */

addToControlBuffer(3);

if ((i & 8) == 8)  /* knapp 4? */

addToControlBuffer(4);

if ((i & 16) == 16) /* knapp 5? */

/*		addToControlBuffer(5);*/

game_start();

if ((i & 32) == 32) /* knapp 6? */

addToControlBuffer(6);

}

void addToControlBuffer(int num) {

control_unit[num-1] = TRUE;

}

void resetControls(void) { /* körs när buffern lästs av */

int i;

for(i=0; i<6; i++)

control_unit[i] = FALSE;

}

int isButtonPushed(int num) {

return (control_unit[num-1]);

}

void eeprom_write(int rel_adr, int num) {

int i;

/*	*(unsigned char *)(0xa000+rel_adr) = (unsigned char)num; */

eeprom[rel_adr] = num;

for(i=0;i<0x1111;i++);

}

int eeprom_read(int rel_adr) {

int i, num;

/*	num = *(unsigned char *)(0xa000+rel_adr); */

num = eeprom[rel_adr];

/*	for(i=0;i<0x1111;i++); */

return num;

}

### display.c

/*

Displayens två delar är vardera 64x64 pixlar eller 8x8 tecken,

där varje tecken är 8x8 pixlar.

Y är positivt i riktningen uppåt och X positivt åt höger.

Med Page-Address menas X-position räknat i tecken (0-7 st)

Med Y-Address menas Y-position i pixlar (0-63)

När en byte skrivs gäller: bit 0 är längst till vänster och

bit 7 till höger (omvänt med andra ord)

--

I funktionerna som skriver ut tecken används:

X positivt höger, Y positivt nedåt.

(1,1) är längst uppe till vänster och (8, 16) är längst ner

till höger

--

observera att det krävs en kort paus efter vissa instruktioner

till displayen

*/

void display_on(void);

void display_clear(void);

void display_update(void); /* inget skrivs till displayen förrän denna körs */

void display_writechar(int x, int y, char *tecken);

void display_inversewritechar(int x, int y, char *tecken);

void waitForDisplay(void);

void eeprom_write(int rel_adr, int num);

int eeprom_read(int rel_adr);

char kloss_tecken[8] = { 0xfe, 0x83, 0x83, 0x83, 0x83, 0x83, 0xff, 0x7f };

char inget_tecken[8] = { 0, 0, 0, 0, 0, 0, 0, 0 };

void display_on(void) {

display1_instr = 0x3f; /* sätt på displayen */

display2_instr = 0x3f;

display1_instr = 0xc0; /* ställ in översta raden */

display2_instr = 0xc0;

display_clear(); /* töm displayen */

display_update();

}

void display_clear() {

int x, y;

for(x=0;x<8;x++)

for(y=0;y<128;y++)

disp_mem[x*128 + y] = 0;

}

void display_update(void) {

int x, y;

display1_instr = 0x40; /* sätt Y=0, denna ökas med 1 varje gång en byte skrivs */

display2_instr = 0x40;

for (x=0; x<8; x++) {

display1_instr = 0xb8 + x; /* sätt x-position */

display2_instr = 0xb8 + x;

for (y=0; y<8*8; y++) {

display1_data = disp_mem[x*128 + 127-y];

display2_data = disp_mem[x*128 + 63-y];

}

}

}

void display_writechar(int x, int y, char *tecken) {

int m;

for(m=0; m < 8; m++) {

disp_mem[ (x-1) * 128 + (y-1)*8 + m ] = tecken[m];

}

}

void display_inversewritechar(int x, int y, char *tecken) {

int m;

for(m=0; m < 8; m++) {

disp_mem[ (x-1) * 128 + (y-1)*8 + m ] = 0xff - (tecken[m] * 2);

}

}

void waitForDisplay() {

int i;

/* vänta typ en klockpuls, så gör nåt kort */

i++;

}

### game.c

/*

att göra:

* spara i eeprom

* ordna vettiga tabeller för poäng och hastighet

enligt specs: rad bort=10p, kloss ner=1p, 200p=nu nivå

*/

#define HORIZ_WAIT 4

#define ROTATE_WAIT 4

#define EMPTY 0

#define FULL 1

#define SCORE_FOR_PLACED_BLOCK	4

#define SCORE_FOR_FULL_ROW	25

void moveBlock();				/* flytta klossen enligt vilka knappar som tryckts ned */

void newBlock();				/* välj ny kloss */

void newGame();				/* initiera för nytt spel */

void drawBlock();				/* rita kloss */

void eraseBlock();			/* ta bort kloss */

void placeBlock();			/* lägg klossen och välj ny */

void menu();					/* Skriv ut menyn */

void setMenu();				/* Ändrar menyn */

void showHighscore();		/* Visar highscorelistan */

void printScore(int y, int sc);

void redrawBoard();

void removeFullRows(void);

void pauseGame();				/* Pausar spelet */

void addScore(int);			/* Lägg till poäng och uppdatera eventuellt hastigheten */

void writeHighScore();

void readHighScore();

void updateHighScore(char *name, int sc);

int isBlockInsideX();		/* kolla så att klossen får plats i ramen */

int isBlockInsideY();

void setHighscore(int score);

int	currentX,		/* X- och Y-position för denna kloss mittpunkt */

currentY;

int	currentType,	/* klossens typ 0-5 */

currentAngle;	/* klossens vinkel 0-3 */

int	gameCounter,	/* räknar tiden sedan spelet startades */

buttonCounter,	/* räknar antalet gånger en knapp tryckts ned */

vertCounter,	/* räknar tiden sedan klossen flyttades ned sist */

horizCounter,	/* och flyttades i sidled sist */

rotateCounter,	/* tiden sen rotation */

menuCounter,	/* räknare för menyn */

gameOver,		/* är spelet slut? */

highscore10,	/* Tionde bästa poängen*/

gameSpeed;		/* Hastighet 0-10, 10 snabbast möjliga */

int	score;			/* poängställning */

int	highscore[10][4];	/* hiscore-lista */

int	mode;							/*0=meny, 1=spel, 2=highscore, 3=set highscore, 4=paus*/

int	gameBoard[9][17];			/* spelplanen. X:1-8 Y:1-16 */

int	kloss[7][4][4][2] =	{	/* Kloss-nr, klossens rotation, klossens bitar, x- & y-värde */

{{{1,0}, {2,0}, {1,1}, {1,2}},

{{0,0}, {1,0}, {2,0}, {2,1}},

{{0,2}, {1,2}, {1,1}, {1,0}},

{{0,0}, {0,1}, {1,1}, {2,1}}},

{{{0,0}, {1,0}, {1,1}, {1,2}},

{{0,1}, {1,1}, {2,1}, {2,0}},

{{1,0}, {1,1}, {1,2}, {2,2}},

{{0,0}, {0,1}, {1,0}, {2,0}}},

{{{0,0}, {-1,0}, {0,1}, {1,0}},

{{0,0}, {-1,0}, {0,-1}, {0,1}},

{{0,0}, {-1,0}, {1,0}, {0,-1}},

{{0,0}, {1,0}, {0,1}, {0,-1}}},

{{{0,0}, {0,1}, {-1,0}, {-1,-1}},

{{0,0}, {-1,0}, {0,-1}, {1,-1}},

{{0,0}, {0,1}, {-1,0}, {-1,-1}},

{{0,0}, {-1,0}, {0,-1}, {1,-1}}},

{{{0,0}, {0,1}, {1,0}, {1,-1}},

{{0,0}, {0,-1}, {-1,-1}, {1,0}},

{{0,0}, {0,1}, {1,0}, {1,-1}},

{{0,0}, {0,-1}, {-1,-1}, {1,0}}},

{{{0,0}, {0,1}, {0,-1}, {0,-2}},

{{0,0}, {1,0}, {-1,0}, {-2,0}},

{{0,0}, {0,1}, {0,-1}, {0,-2}},

{{0,0}, {1,0}, {-1,0}, {-2,0}}},

{{{0,0}, {0,1}, {1,0}, {1,1}},

{{0,0}, {0,1}, {1,0}, {1,1}},

{{0,0}, {0,1}, {1,0}, {1,1}},

{{0,0}, {0,1}, {1,0}, {1,1}}},

};

char A[8] = { 0x04, 0x0A, 0x11, 0x11, 0x1F, 0x11, 0x11, 0x00 };

char B[8] = { 0x0F, 0x11, 0x11, 0x0F, 0x11, 0x11, 0x0F, 0x00 };

char C[8] = { 0x0E, 0x11, 0x01, 0x01, 0x01, 0x11, 0x0E, 0x00 };

char D[8] = { 0x07, 0x09, 0x11, 0x11, 0x11, 0x09, 0x07, 0x00 };

char E[8] = { 0x0F, 0x01, 0x01, 0x07, 0x01, 0x01, 0x0F, 0x00 };

char F[8] = { 0x0F, 0x01, 0x01, 0x07, 0x01, 0x01, 0x01, 0x00 };

char G[8] = { 0x0E, 0x11, 0x01, 0x01, 0x1D, 0x11, 0x0e, 0x00 };

char H[8] = { 0x09, 0x09, 0x09, 0x0F, 0x09, 0x09, 0x09, 0x00 };

char I[8] = { 0x08, 0x08, 0x08, 0x08, 0x08, 0x08, 0x08, 0x00 };

char J[8] = { 0x0F, 0x08, 0x08, 0x08, 0x08, 0x09, 0x06, 0x00 };

char K[8] = { 0x11, 0x09, 0x05, 0x03, 0x05, 0x09, 0x11, 0x00 };

char L[8] = { 0x01, 0x01, 0x01, 0x01, 0x01, 0x01, 0x0F, 0x00 };

char M[8] = { 0x63, 0x55, 0x49, 0x41, 0x41, 0x41, 0x41, 0x00 };

char N[8] = { 0x11, 0x13, 0x13, 0x15, 0x19, 0x19, 0x11, 0x00 };

char O[8] = { 0x0E, 0x11, 0x11, 0x11, 0x11, 0x11, 0x0E, 0x00 };

char P[8] = { 0x0F, 0x11, 0x11, 0x0F, 0x01, 0x01, 0x01, 0x00 };

char Q[8] = { 0x0e, 0x11, 0x11, 0x11, 0x11, 0x11, 0x2e, 0x00 };

char R[8] = { 0x0F, 0x11, 0x11, 0x0F, 0x05, 0x09, 0x11, 0x00 };

char S[8] = { 0x1E, 0x01, 0x01, 0x0E, 0x10, 0x10, 0x0F, 0x00 };

char T[8] = { 0x1F, 0x04, 0x04, 0x04, 0x04, 0x04, 0x04, 0x00 };

char U[8] = { 0x11, 0x11, 0x11, 0x11, 0x11, 0x11, 0x0E, 0x00 };

char V[8] = { 0x11, 0x11, 0x11, 0x0A, 0x0A, 0x0A, 0x04, 0x00 };

char W[8] = { 0x41, 0x41, 0x41, 0x41, 0x49, 0x55, 0x63, 0x00 };

char X[8] = { 0x11, 0x11, 0x0A, 0x04, 0x0A, 0x11, 0x11, 0x00 };

char Y[8] = { 0x11, 0x0A, 0x04, 0x04, 0x04, 0x04, 0x04, 0x00 };

char Z[8] = { 0x1F, 0x10, 0x08, 0x04, 0x02, 0x01, 0x1F, 0x00 };

char k1[8] = { 0x10, 0x18, 0x10, 0x10, 0x10, 0x10, 0x10, 0x00 };

char k2[8] = { 0x0E, 0x11, 0x08, 0x04, 0x02, 0x01, 0x1F, 0x00 };

char k3[8] = { 0x0E, 0x11, 0x10, 0x0C, 0x10, 0x11, 0x0E, 0x00 };

char k4[8] = { 0x09, 0x09, 0x09, 0x0F, 0x08, 0x08, 0x08, 0x00 };

char k5[8] = { 0x0F, 0x01, 0x01, 0x07, 0x08, 0x09, 0x06, 0x00 };

char k6[8] = { 0x0C, 0x02, 0x01, 0x0F, 0x11, 0x11, 0x0E, 0x00 };

char k7[8] = { 0x1F, 0x10, 0x08, 0x08, 0x04, 0x04, 0x02, 0x00 };

char k8[8] = { 0x0E, 0x11, 0x11, 0x0E, 0x11, 0x11, 0x0E, 0x00 };

char k9[8] = { 0x0E, 0x11, 0x11, 0x1E, 0x10, 0x08, 0x06, 0x00 };

char k0[8] = { 0x18, 0x24, 0x42, 0x42, 0x42, 0x24, 0x18, 0x00 };

char star[8] = { 0x92, 0x54, 0x38, 0xfd, 0x38, 0x54, 0x92, 0x00 };

char line[8] = { 0x00, 0x7f, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00 };

char blank[8] = { 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00, 0x00 };

int scoreLevel[] = {	/* poängen som krävs för att nå upp en nivå */

50,			/* speed = 0 */

300,

800,

1600,

3200,

6400,			/* speed = 5 */

12800,

25600,

51200,

60000,

0				/* speed = 10 */

};

unsigned int vertWait[] = { /* snabbheten vid varje nivå */

32,

27,

22,

16,

14,

12,

10,

5,

3,

1,

0,

};

void game_start() {

int i;

int menuMode;					/*0=spela, 1=highscore*/

mode = 0;

menuMode = 0;

score=0;

setMenu();

display_writechar(2,8,star );

display_writechar(2,9,blank );

display_update();

do {

resetControls();

do {

/* vänta på knapptryckning */

check_buttons();

} while (!isButtonPushed(1) && !isButtonPushed(2) && !isButtonPushed(6));

if (isButtonPushed(1) || isButtonPushed(2))

menuMode = (menuMode +1) % 2;

if (isButtonPushed(6)) {

if (menuMode == 0) {

mode = 1;

newGame();

} else {

mode = 2;

showHighscore();

}

setMenu();

}

/* uppdatera förändringar */

if (menuMode == 0) {

display_writechar(2,8,star );

display_writechar(2,9,blank );

} else {

display_writechar(2,8,blank );

display_writechar(2,9,star );

}

display_update();

for(i=0;i<0x8000;i++); /* vänta lite */

} while (mode == 0);

}

void setMenu() {

display_clear();

display_writechar(2,2,T );

display_writechar(3,2,E );

display_writechar(4,2,T );

display_writechar(5,2,R );

display_writechar(6,2,I );

display_writechar(7,2,S );

display_writechar(4,3,A );

display_writechar(5,3,V );

display_writechar(2,4,C );

display_writechar(3,4,A );

display_writechar(4,4,L );

display_writechar(5,4,L );

display_writechar(6,4,E );

display_writechar(3,5,O );

display_writechar(4,5,C );

display_writechar(5,5,H );

display_writechar(3,6,A );

display_writechar(4,6,N );

display_writechar(5,6,N );

display_writechar(6,6,A );

display_writechar(4,8,S );

display_writechar(5,8,P );

display_writechar(6,8,E );

display_writechar(7,8,L );

display_writechar(8,8,A );

display_writechar(4,9,T );

display_writechar(5,9,O );

display_writechar(6,9,P );

display_writechar(7,9,k1 );

display_writechar(8,9,k0 );

display_update();

}

void menu() {

}

void showHighscore() {

int i;

mode = 2;

display_clear();

display_writechar(1,1,k1 );

display_writechar(2,1,k0 );

display_writechar(4,1,B );

display_writechar(5,1,A );

display_writechar(6,1,S );

display_writechar(7,1,T );

display_writechar(8,1,A );

/* Skriv ut listan */

readHighScore();

for(i=0;i<10;i++) {

display_writechar(1, 3+i, A+8*(highscore[i][0] - 0x41));

display_writechar(2, 3+i, A+8*(highscore[i][1] - 0x41));

display_writechar(3, 3+i, A+8*(highscore[i][2] - 0x41));

printScore(3+i, highscore[i][3]);

}

display_writechar(1,15,star );

if (score != 0) {

display_writechar(1,13,D );

display_writechar(2,13,I );

display_writechar(3,13,N );

display_writechar(4,13,P );

display_writechar(5,13,O );

display_writechar(6,13,A );

display_writechar(7,13,N );

display_writechar(8,13,G );

printScore(14, score);

score = 0;

}

display_writechar(3,15,T );

display_writechar(4,15,I );

display_writechar(5,15,L );

display_writechar(6,15,L );

display_writechar(5,16,M );

display_writechar(6,16,E );

display_writechar(7,16,N );

display_writechar(8,16,Y );

display_update();

resetControls();

for(i=0;i<0x5555;i++); /* vänta lite */

do {

for(i=0;i<0x1000;i++)

check_buttons();

if (isButtonPushed(6)) {

mode = 0;

}

resetControls();

} while (mode == 2);

}

void moveBlock() {

int oldX, oldY, oldAngle;

if (mode == 1) {

oldAngle=currentAngle;

oldX=currentX;

oldY=currentY;/*	De tre räknarna ser till så att inte förflyttning eller vändande på klossen kan ske för ofta	*/

/* flytta klossen efter vilka knappar som tryckts ned */

if (isButtonPushed(1) && rotateCounter > ROTATE_WAIT ) { 	  /* upp */

currentAngle=(currentAngle+1) & 3;

rotateCounter = 0;

buttonCounter++;

}

if (isButtonPushed(2)) { /* ner */

currentY++;

buttonCounter++;

}

if (isButtonPushed(3) && horizCounter > HORIZ_WAIT ) { /* vänster */

currentX--;

horizCounter = 0;

buttonCounter++;

}

if (isButtonPushed(4) && horizCounter > HORIZ_WAIT ) { /* höger */

currentX++;

horizCounter = 0;

buttonCounter++;

}

if (isButtonPushed(6) && horizCounter > HORIZ_WAIT ) {

pauseGame();

horizCounter = 0;

}

if (vertCounter > vertWait[gameSpeed]) {								/* efter en viss tid flyttas blocket nedåt*/

vertCounter = 0;

if (!isButtonPushed(2))         /* men inte två steg åt gången så om spelaren också tryckt nedåt görs inte detta */

currentY++;

}

/* kolla nu om flytten var giltig, annars gå tillbaka till som det var */

/* ifall klossen flyttas både neråt och åt sidan blir det specialfall */

if (!isBlockValid()) {										/* flytten går ej */

if (currentX != oldX && currentY != oldY) {		/* ändringar både i X- och Y-led? */

/*om klossen krockar med en annan, flytta först tillbaka i Y-led och prova igen och

sen i X-led om den fortfarande krockar*/

currentY = oldY;

if (!isBlockValid())

currentX = oldX;

} else {														/* flyttades antingen i sidled eller nedåt */

if (currentX != oldX) {								/* flyttades i sidled */

currentX = oldX;									/* men det gick inte, så gå tillbaka */

} else if (currentY != oldY) {					/* flyttades nedåt */

/* klossen fastnar i nedre kanten eller krockar med annan */

/* så den stannar där */

currentY = oldY;

placeBlock();										/* placera klossen där */

/* Kollar om någon rad är full */

removeFullRows();

} else {													/* roterades */

currentAngle = oldAngle;

}

}

}

resetControls();

vertCounter++;

horizCounter++;

gameCounter++;

rotateCounter++;

}

}

void newBlock() {

int i, Ycoord=1000;							/* försök slumpa ut nåt så använd variabler som förändras ofta */

currentType =	(horizCounter +

vertCounter +

rotateCounter +

score +

gameCounter +

buttonCounter) & 7;

if (currentType == 7)

currentType = 0;							/* currentType ska vara mellan 0 och 6 */

currentAngle = 0;

currentX = 4;

/*

Måste ta reda på koordinaten för klossens översta del så att hela

klossen placeras så långt upp som möjligt

*/

for(i=0;i<4;i++)

if (kloss[currentType][currentAngle][i][1] < Ycoord)

Ycoord=kloss[currentType][currentAngle][i][1];

currentY = 1-Ycoord;						/* placeringen beror på klossens form */

/* om det är klossar i vägen är spelet slut */

if (!isBlockValid())

gameOver = TRUE;

}

void newGame() {

int x, y, i;

mode = 1;

vertCounter = 0;

horizCounter = 0;

rotateCounter = 0;

gameCounter = 0;

buttonCounter = 0;

gameSpeed = 0;

score = 0;

gameOver = FALSE;

for(x=1;x<=8;x++)

for(y=1;y<=16;y++)

gameBoard[x][y] = EMPTY;

display_clear();

resetControls();

for(i=0;i<0x5555;i++); /* vänta lite */

newBlock();

do {

drawBlock();

printScore(1, score);

display_update();

eraseBlock();

check_buttons();

moveBlock();

} while (!gameOver);

mode=2;

if (score > highscore10) {

mode=3;

setHighscore(score);

} else {

showHighscore();

}

}

void setHighscore(int score) {

int charplace;

char nameStr[3];

char *name = nameStr;

int i;

mode=3;

display_clear();

display_writechar(2,2,S );

display_writechar(3,2,K );

display_writechar(4,2,R );

display_writechar(5,2,I );

display_writechar(6,2,V );

display_writechar(3,3,D );

display_writechar(4,3,I );

display_writechar(5,3,T );

display_writechar(6,3,T );

display_writechar(3,4,N );

display_writechar(4,4,A );

display_writechar(5,4,M );

display_writechar(6,4,N );

display_writechar(3,8,line );

display_writechar(4,8,line );

display_writechar(5,8,line );

display_writechar(3,14,T );

display_writechar(4,14,I );

display_writechar(5,14,L );

display_writechar(6,14,L );

display_writechar(4,15,B );

display_writechar(5,15,A );

display_writechar(6,15,S );

display_writechar(7,15,T );

display_writechar(8,15,A );

display_update();

charplace=0;

for(i=0;i<3;i++) {

name[i] = 0;

display_writechar(i+3, 7, A+8*name[i]);

}

display_inversewritechar(charplace+3, 7, A+8*name[charplace]);

do {

for(i=0;i<0x1000;i++) {

check_buttons();

}

if (mode==3) {

if (isButtonPushed(1)) {

name[charplace]++;

if (name[charplace]>25)

name[charplace]=0;

display_inversewritechar(3+charplace, 7, A+8*name[charplace]);

display_update();

}

if (isButtonPushed(2)) {

name[charplace]--;

if (name[charplace] < 0 || name[charplace] > 25)	/* unsigned jämförelse */

name[charplace]=25;

display_inversewritechar(3+charplace, 7, A+8*name[charplace]);

display_update();

}

if (isButtonPushed(3)) {

display_writechar(3+charplace, 7, A+8*name[charplace]);

charplace--;

if (charplace < 0)

charplace = 0;

display_inversewritechar(3+charplace, 7, A+8*name[charplace]);

display_update();

}

if (isButtonPushed(4)) {

display_writechar(3+charplace, 7, A+8*name[charplace]);

charplace++;

if (charplace > 2)

charplace = 2;

display_inversewritechar(3+charplace, 7, A+8*name[charplace]);

display_update();

}

if (isButtonPushed(6)) {

mode = 2;

/* Skriv ändringar till listan */

/*xxx*/

updateHighScore(name, score);

showHighscore();

}

resetControls();

}

} while (mode == 3);

}

void drawBlock(void) {

int i;

for(i=0;i<4;i++)

display_writechar(	currentX+kloss[currentType][currentAngle][i][0],

currentY+kloss[currentType][currentAngle][i][1],

kloss_tecken

);

}

void eraseBlock(void) {

int i;

for(i=0;i<4;i++)

display_writechar(	currentX+kloss[currentType][currentAngle][i][0],

currentY+kloss[currentType][currentAngle][i][1],

inget_tecken

);

}

void placeBlock(void) {

int i;

for(i=0;i<4;i++)

gameBoard[	currentX+kloss[currentType][currentAngle][i][0] ][

currentY+kloss[currentType][currentAngle][i][1] ] = FULL;

drawBlock();

addScore( SCORE_FOR_PLACED_BLOCK );

newBlock();

}

int isBlockInsideX() {												/* kolla om klossen är innanför vänstra och högra ramen */

int ok = TRUE, i;

for(i=0;i<4;i++)

if (	currentX+kloss[currentType][currentAngle][i][0] < 1 ||

currentX+kloss[currentType][currentAngle][i][0] > 8)

ok = FALSE; /* denna delklossen är utanför */

return (ok);

}

int isBlockInsideY() {												/* kolla om klossen är ovanför undre ramen */

/* obs! denna procedur kollar inte om klossen är ovanför övre ramen */

int ok = TRUE, i;

for(i=0;i<4;i++)

if ( currentY+kloss[currentType][currentAngle][i][1] > 16)

ok = FALSE;													/* denna delklossen är utanför */

return (ok);

}

int isBlockValid() {													/* kolla om klossen krockar med en annan, isåfall är flytten ogiltig */

int ok = TRUE, i;

/* Kolla först så klossen är innanför spelplanen */

if (!isBlockInsideX())											/* kolla först så klossen är innanför skärmen */

return FALSE;

if (!isBlockInsideY())

return FALSE;

for(i=0;i<4;i++)

if (gameBoard[ currentX+kloss[currentType][currentAngle][i][0] ][

currentY+kloss[currentType][currentAngle][i][1] ] != EMPTY )

ok = FALSE; /* denna platsen är upptagen */

return (ok);

}

void printScore(int y, int sc) {

int i;

int digit, scoreP;

scoreP = sc;

i = 8;

do {

digit = scoreP % 10;

scoreP = scoreP / 10;

switch (digit) {

case 0: display_writechar(i,y,k0 ); break;

case 1: display_writechar(i,y,k1 ); break;

case 2: display_writechar(i,y,k2 ); break;

case 3: display_writechar(i,y,k3 ); break;

case 4: display_writechar(i,y,k4 ); break;

case 5: display_writechar(i,y,k5 ); break;

case 6: display_writechar(i,y,k6 ); break;

case 7: display_writechar(i,y,k7 ); break;

case 8: display_writechar(i,y,k8 ); break;

case 9: display_writechar(i,y,k9 ); break;

}

i--;

} while (scoreP > 0);

}

void removeFullRows(void) {

int x, y, fullRow;

int n, m;

/*

först kollar vi om översta raden är tom, isåfall tas den bort utan att nåt flyttas

de andra raderna behandlar vi lite annorlunda.

*/

fullRow = TRUE; /* vi antar att raden är full och kollar om det stämmer*/

for(x=1;x<=8;x++)

if (gameBoard[x][1] == EMPTY) /* nej raden är inte full */

fullRow = FALSE;

if (fullRow) { /* översta raden full */

for(x=1;x<=8;x++) /* så ta bort alla rutor */

gameBoard[x][1] = EMPTY;

redrawBoard(); /* något har ändrats så rita om */

display_update();

for(n=1;n<0x1111;n++); /* vänta lite */

}

for(y=2;y<=16;y++) { /* kolla alla rader utom första; den går inte att ta bort på detta sätt */

fullRow = TRUE;

for(x=1;x<=8;x++) /* alla 8 rutor i bredd */

if (gameBoard[x][y] == EMPTY)

fullRow = FALSE;

if (fullRow) {

/* ta bort rad nr. y */

addScore( SCORE_FOR_FULL_ROW );

for(m=y;m>=2;m--) /* flytta ner alla rader ovanför ett steg, och börja flytta nerifrån */

for(n=1;n<=8;n++) /* alla 8 rutor i bredd */

gameBoard[n][m] = gameBoard[n][m-1]; /* kopiera ovanstående ruta */

for(n=1;n<=8;n++)

gameBoard[n][1] = EMPTY; /* ta bort översta raden */

redrawBoard(); /* något har ändrats så rita om */

display_update();

for(n=1;n<0x1111;n++); /* vänta lite */

}

}

}

void redrawBoard() {

int x, y;

/* vi måste rita om allt */

display_clear();

for(y=1;y<=16;y++) /* hela brädet */

for(x=1;x<=8;x++)

if (gameBoard[x][y] == FULL) /* finns här en kloss? */

display_writechar(x, y, kloss_tecken); /* rita den isåfall */

printScore(1, score); /* rita ut poäng */

}

void pauseGame() {

int i;

resetControls();

for(i=0;i<0x5555;i++); /* vänta lite */

mode = 4;

display_writechar(1, 7, star);

display_writechar(2, 7, P);

display_writechar(3, 7, A);

display_writechar(4, 7, U);

display_writechar(5, 7, S);

display_writechar(6, 7, E);

display_writechar(7, 7, D);

display_writechar(8, 7, star);

display_update();

do {

for(i=0;i<0x1000;i++)

check_buttons();

if (isButtonPushed(6)) {

mode = 1;

}

resetControls();

} while (mode == 4);

redrawBoard();

}

void addScore(int bonus) {

/* öka poäng beroende på snabbhet */

score += bonus * (gameSpeed+1);

/* går ej snabbare än speed = tio annars öka om spelaren fått tillräckligt med poäng */

if ( gameSpeed < 10 && score >= scoreLevel[gameSpeed] )

gameSpeed++;

}

void writeHighScore() {

int i;

for(i=0;i<10;i++) {

eeprom_write(5*i, highscore[i][0]);

eeprom_write(5*i+1, highscore[i][1]);

eeprom_write(5*i+2, highscore[i][2]);

eeprom_write(5*i+3, highscore[i][3] / 0x100 );

eeprom_write(5*i+4, highscore[i][3] % 0x100 );

}

}

void readHighScore() {

int i;

for(i=0;i<10;i++) {

highscore[i][0] = eeprom_read(5*i); /* initialer */

highscore[i][1] = eeprom_read(5*i+1);

highscore[i][2] = eeprom_read(5*i+2);

highscore[i][3] = eeprom_read(5*i+3) * 0x100 + eeprom_read(5*i+4); /* poängen */

}

}

void updateHighScore(char *name, int sc) {

int i, newRow;

newRow=10;

for(i=9;i>=0;i--)

if (sc >= highscore[i][3])

newRow=i;

if (newRow < 10) {

/* flytta ner */

for(i=9;i>newRow;i--) {

highscore[i][0] = highscore[i-1][0];

highscore[i][1] = highscore[i-1][1];

highscore[i][2] = highscore[i-1][2];

highscore[i][3] = highscore[i-1][3];

}

highscore[newRow][0] = name[0] + 0x41;

highscore[newRow][1] = name[1] + 0x41;

highscore[newRow][2] = name[2] + 0x41;

highscore[newRow][3] = sc;

}

writeHighScore();

}

### avben.68k

**********************************************************************

*                            avben.68k

*                            ---------

*           Ett program som tillater avbrottsniva 4 och hogre.

*           Anropas fran tetris.c med funktionen _avben().

*

***********************************************************************

xdef 	__avben       	   Definierar ett lage __avben.

__avben	equ 	*

move	#$2300,SR          Tillåt endast avbrott 4+

rts			   atervand till tetris.c

end

### exp2.68k

**********************************************************************

*                          exp2.68k

*                          --------

*           Avbrottshanterare till avbrottsniva 2

*

***********************************************************************

xdef 	__exp2       	   Definierar ett lage __exp2.

xref  	_exp2		   Refererar till lage _exp2.

__exp2	equ 	*

movem.l	D0-D7/A0-A7,-(A7)   Spara alla register!!

lea.l   $0030401a,A0	    Endast for PI/T.

move.b  #01,(A0)	    Endast for PI/T.

jsr	_exp2               Hoppa till _exp2 i tetris.c.

movem.l (A7)+,D0-D7/A0-A7   Aterstall ala register.

rte 			    Avsluta avbrott.

End

### pmain.68k

***************************************************************************

*                        pmain.68k

*                        ---------

* 	 Detta assemblyprogram

* 	 initierar stackpekaren,

* 	 den globala datapekaren {data},

* 	 avbrottsvektorn,

* 	 nollstaller alla register

* 	 samt gor ett subrutinanrop till _main.

*

***************************************************************************

xref	_main		Refererat till symbolen _main = main i tetris.c

M_main	equ	*

section	init

xdef	__main		Definierar __main

__main

xref	data		Segmentet data som ar den globala data-arean.

xref	__exp2          Symbolen for avbrottshanteraren vid avbrott 4

movea.l	#$00007ffc,A7	Stackpekare till 7ffc.

move	#$2700,SR	Supervisor och inga avbrott tillatna.

lea	data,A5		ladda A5 med global datapekare. ETT MASTE!!

suba.l	A6,A6		Nollstall frame pointer.

lea.l 	$00000068,A4	Ladda avbrottsvektorn, nr 2 på address 0x68

move.l	#__exp2,(A4)

suba.l	A4,A4

suba.l	A3,A3

suba.l	A2,A2

suba.l	A1,A1

suba.l	A0,A0

clr.l	D7

clr.l	D6

clr.l	D5

clr.l	D4

clr.l	D3

clr.l	D2

clr.l	D1

clr.l	D0

jsr	_main		Hoppa till _main vilket ar i filen tetris.c

* Om nagot gar fel sa kommer programmet hit vid ett return fran _main

* och hamnar i en evig loop.

errloop

jmp	errloop

*  Slut pa inits.

*

*  Org kommandot ar ett direktiv till assemblern, att allt efterfoljande

*  skall borja laggas pa den adress som anges. I detta fall ar det SP med

*  borjan pa adress 0 (dc.l => 4 byte) och sedan vardet pa PC.

*

org	$0

dc.l	$00007ffc

dc.l	__main

*  Avbrottsvektorerna far ratt varde som ovan. Detta anvands nar man vill

*  branna sitt PROM. Det gar inte att gora som ovan, ty det ar inte mojligt

*  att skriva i PROM.

*

*	org     $68

*	dc.l    __exp2		PI/T on it68 board

* Nedanstaende definierar __main som startadress.

end	__main

# Appendix D – how to play the game

| TETRIS AV CALLE OCH ANNA * SPELA TOP10 | This is the main menu. The star is moved with the and ¯ buttons. When button 2 is pressed you choose the thing that the star is in front of. When the game is started you move the block to left, right and down with the buttons ¬, ® and ¯, to rotate a block you press . To pause the game you press button 2 and to start it again you press that button again. When the game is finished you have to write your name if you get a better result than the ten best have had. When you write your name you browse the letters with and ¯. To go to the next place you press ®. When you have finished writing your name you press button 2. If you not are one of the ten best you see the top 10 list and in the bottom of that you can also see your own score. To go from the top 10 list to the main menu you press button 2. The game will restart if you press button 1. The game starts slow and then increase more and more while you are playing. The higher speed the game has the more point do you get for each thing that is done. If you think that the block moves to many steps at a time, you are pressing the button for too long. |
| --- | --- |